QtCRE Software Upgrade Instructions

- Make sure that the small switch on the side of the QtCRE is set to the “Down” position.  This enables the external USB jack.

- Upload the hex file from csmqt.com/support to a new or empty USB Flash Drive.

- Unplug the Qt-CRE.

- Place the USB stick with the Hex file into the USB port of the QtCRE.

- Plug in the Qt-CRE.

- The backlight on the blank LCD display will illuminate.

- Rapid flashing will turn to a slow flash.

- Once the LCD flashing ends,  “System Booting    FW 2.x.x” will appear on the display.

- After four seconds, “Cambridge Sound Management” will be displayed.

- After an additional four seconds, “Masking Off” will be displayed.

- Remove the USB stick from the USB port.

- Make sure that the small switch on the side of the QtCRE is restored to the “Up” position.  This enables the privacy signs to operate.